the davies-young company

2700 Wagner Place, Maryland Heights, Missouri 63043

Material Safety Data

| N.F.P.A. 4 - Extreme | L h HEALTH | 0 |
| --- | --- | --- |
| 3 = High 2 = Moderate | iQ^- | 0 |
| 1 = Slight 0 = Insignificant | X^V REACTIVITY | 0 |

Emergency Telephone:314/291-1900(8a.m. -5p.m.)314/344-2077(5p.m. -8a.m.)

| SECTION 1 - IDENTIFICATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PRODUCT NAME BUCKEYE CITATION |  | DATE PREPARED April 26, 1989 |  |  |  |
| CHEMICAL FAMILY Floor Finish |  | CODE 5115 |  |  |  |
| PROPER D.O.T. shipping name Polishing or Wax Compound |  |  |  |  |  |
| D.O.T. HAZARD Not Clarified CLASSIFICATION NC" " "n'3a |  |  |  |  |  |
| SECTION II -HAZARDOUS INGREDIENTS (OSHA 29 CFR 1910.1200) |  |  |  |  |  |
| % BY WEIGHT | MATERIAL |  | T.L.V. |  | C.A.S. NO. |
|  | None |  |  |  |  |
| SECTION II- B - NON-HAZARDOUS INGREDIENTS |  |  |  |  |  |
| % BY WEIGHT RANGE | MATERIAL |  |  |  | C.A.S.NO. |
| 52.9-43 | Water |  |  |  | - |
| 35-37 | Acrylic Emulsion |  |  |  | Trade Secret |
| 3-4 | Acrylate Copolymer |  |  |  | Trade Secret |
| 4-6 | Polyethylene Emulsion |  |  |  | Blend |
| 4-6* | Diethylene Glycol Ethyl Ether* |  |  |  | 111-90-0 |
| 1-3 | Tributoxy Ethyl Phosphate |  |  |  | 78-51-3 |
| 0.1-1.0 | Leveling Aids Defoaming Agent, Preservative |  |  |  |  |
| ITEMS MARKED 'ARE SARA TITLE III REPORTABLE |  |  |  |  |  |
| SECTION III - PHYSICAL DATA |  |  |  |  |  |
| BOILING POINT°F | 212 °F | pH (CONC.) | 8.8 |  |  |
| SOLUBILITY IN WATER | Miscible | pH (USE DILUTION) | N/A |  |  |
| % VOLATILE BY WEIGHT | 83.0 | EVAPORATION RATE (WATER - 1) | 1 | .0 |  |
| SPECIFIC GRAVITY | 1.04 | LIQUID[X} POWDER CD' PASTE fj AEROSOLQ |  |  |  |
| APPEARANCE AND ODOR | White Opaque Liquid Chemical Odor |  |  |  |  |
| SECTION IV - FIR BAND EXPLOSION DA TA |  |  |  |  |  |
| FLASH POINT (TEST METHOD) None (TAG C. C.) |  |  |  | FLAMMABLE LIMITS |  |
| EXTINGUISHING MEDIA Q CO,, Q FOAM Q DRY CHEMICAL Q WATER [ [OTHER |  |  |  | UPPER LOWER |  |
|  |  |  |  | N/A N/A |  |
| SPECIAL FIRE FIGHTING PROCEDURES | None — Not Flammable |  |  |  |  |
| UNUSUAL FIRE AND EXPLOSION HAZARDS | None |  |  |  |  |
| s |  |  |  |  |  |

SECTION V— HEALTH HAZARD DATA

EFFECTS OF OVEREXPOSURE

Eyes: Mild irritant	Inhalation: Headache, nausea

Ingestion:   Nausea       Skin: May cause slight irritation

T.L.V.

Not Determined

PRIMARY ROUTES OF ENTRY

Inhalation

EMERGENCY

FIRST AID PROCEDURES

& Skin:     Flush with water for 15 minutes. If irritation persists, see physician.

Inhalation:         Remove to fresh air.

| Ingestion: Drink large amounts ot water. Call physician. |  |
| --- | --- |
| SECTION VI — REACTIVITY DATA |  |
| STABILITY | STABLE Y CONDITIONS _ 10 avoid None |
|  | UNSTABLE |
| INCOMPATIBILITY | Acids, heavy metal salts and strong alkalies |
| HAZARDOUS DECOMPOSITION PRODUCTS | co2 |
| HAZARDOUS POLYMERIZATION | WILL OCCUR WILL NOT OCCUR X |
| SECTION VII — SPILL OR LEAK PROCEDURES |  |
| response '">'C'< UP W'^ m°Ps anc' rinse area with clear water |  |
| WASTE DISPOSAL „.. . , . . ... .n—™ ,^r^ 1)1 ntfl with arnp amnimtc: nf watpr f i i<?h fn rpwpt |  |

METHOD

SECTION VIII — SPECIAL PROTECTION INFORMA TION

EYE PROTECTION

Safety Glasses when pouring polish

VENTILATION

Adequate to keep vapors dispersed

protection      Rubber Gloves when handling wet polish

SKIN

RESPIRATORY

protection     None

other	None

PROTECTION

SECTION IX— SPECIAL PRECAUTIONS

PRECAUTIONS IN

handling and storage    Keep from freezing. Do not store above 110° F. Keep container closed when not in use.

OTHER PRECAUTIONS

KEEP OUT OF REACH OF CHILDREN

PREPARED BY:

Mark Gindling — Research Specialist

Disclaimer of Liability

As the conditions or methods of use are beyond our control, we do not assume any responsibility and expressly disclaim any liability for any use of the material. Information contained herein is believed to be true and accurate but all statements or suggestions are made without any warranty, express or implied, regarding accuracy of the information, the hazards connected with the use of the iJ3gtfi[i,a,|,,,orth.e.,,results to be obtained from the use thereof.