MATERIAL SAFETY DATA SHEET

(comlies with OSHA Communication Standard 29 CFR 1910.1209 Dept. of Labor)

CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

PRODUCT NAME High Delivery Carb and Choke

Revision Date :  5/10/99

MANUFACTURER/SUPPLIER’S NAME: Hi-Port Aerosol, 9150 Valley View Road, Mandedoneia, OH  44056

TELEPHONE NUMBER (330) 487-4195

Product .		Composition		ACGIH TLV		OSHA PEL

CAS REG NO. UNITS	wgt%		TWA      STEL		TWA    STEL

Toluene

108-88-3		30-40		50 skin     NA		200ppm  NA

Propane-Isobutane-n-Butane

74-98-6		20-30		1000ppm   Asphyxiant           1000ppm  Asphyxiant

Xylene

1330-20-7		20-30		100ppm   150ppm	100ppm  NA

Methanol

67-56-1		1-10		200 skin  250ppm	200ppm NA

Diacetope Alcohol

123-42-2		1-10		50ppm  NA		50ppm	NA

Acetone

67-64-1		1-10		750ppm  1000ppm	1000ppm NE

Methyl Ethyl Ketone

78-93-3		1-10		200ppm  300ppm	200ppm  NA

Additional Component Information:  Acceptable ceiling concentration for toluene – 300 ppm, max peak above ceiling – 8 hour shift –500ppm.  Technical grade xylene contains 18-20% ethyl benzene (100-41-4) which has 100ppm PEL, 100ppm TLV, 125 ppm STEL, and is subject to the reporting requirements of Section 313 of SARA Title III.

HEALTH HAZARD DATA (ACUTE AND CHRONIC)

May cause dizziness or narcosis in high vapor concentrations.  Will cause defatting of skin.  Effects are reversible.  Long term exposure (years) to high concentrations of vapor may cause lung, liver or kidney damage.  The solvents listed have been reported to affect the pentral nervous system.  Aspiration hazard if swallowed.  Eye and skin irritant.  May irritate respiratory tract.  Deliberately concentrating and inhaling the vapor of the contents may be harmful or fatal.

Other:  Diacetone Alcohol may cause amemia.  May cause cardiac abnormalities.  Methanol may cause blindness or death if swallowed.  Methyl Ethyle Ketone may cause embryotoxic effects in laboratory animals.  Overexposure to toluene may cause nasal and brain damage.

Carcinogenicity:  NA.

First Aid:  Give oxygen.  Do not induce vomiting.  Wash eyes and skin with water for 15 minutes.  Get medical help.  Never administer adrenalin following overexposure.

Routes of Entry:  Ingestion, inhalation, and skin

Signs and symptoms of Overexposure:  Inhalation – difficulty in breathing.  Skin – redness.  Ingestion – vomiting

Medial Conditions Aggravated by Exposure:  Heart disease, respiratory disorders

Boiling Point -40F to 289F

Vapor Pressure at 70 degrees F		65

Vapor Density (air=1)	4.0

Evaporation Rate (Butyl Acetate=1)	>1

Specific Gravity (water=1)	0.7

Solubility: nil

Appearance:  Clear

Odor:	Solvent

pH:  Not applicable

Stability:	Stable

Incompatibility:	Strong oxidizers, mineral acids strong alkales

Hazardous Thermal Decomposition/Combustion Products:  Carbon dioxide

Hazardous Polymerization:  Will not occur

California Proposition as:

Warning:  This product contains a chemical known to the State of California to cause birth defects or other reproductive harm.

Ozone depleting substances:  None

HMIS:  Flammability – 4, Reactivity – 0, Health – 3

NFPA:  Flammability – 4, Reactivity – 0, Health – 2

SARA 313 Title IV – Toxic Chemical List:  Methanol (CAS 67-56-1) Methyl Ethyl Ketone (CAS 78-93-3) Toluene (CAS 108-88-3) Xylene (CAS 1330-20-7)

DOT Shipping Name:  Consumer Commodity, ORM-DAerosol, UN 1950

Respiratory Protection Equipment:  Use NIOSH approved respirator if TLV limit is exceeded

Ventilation:  Local exhaust – yes

Protective Gloves:  None required if spraying

Eye and Face Protection:  Safety glasses

Other Protective Equipment:  Long sleeves and long pants

Safe Handling:  Do not smoke while using.  Wash hands after use.

Flash Point:  -142F TCC

Lower Explosion Limit:  1.8

Upper Explosion Limit:  9.5

Extinguishing Media:  Water fog, carbon dioxide, foam, dry chemical

Unusual Fire and Explosion Hazards:  Heated cans my burst.

Special Fire Fighting Procedures:  Keep containers cool.  Use equipment or shielding required to protect against bursting or venting containers.

Spill Procedures:  Use absorbent sweeping compound to soak up materials.  Put into container.  Dispose as hazardous waste.

Waste Disposal Methods:  Dispose as hazardous waste in accordance with EPA RCRA

Precautions to be taken in handling and storage:  Keep away from heat, sparks, or open flame.  Store at Temperatures below 120 degrees F.

Other Information:  When spraying more than one half can continuously or more then one can consecutively use NIOSH NP 300 level.