MSDS#: 9675 Status: Current Status Date: 4/27/01

Product Name: C.P. SOLVENT MASTER Internal MSDS#: 714

Current Company Information:

U.N.X. INCORPORATED

707 ARLINGTON BLVD.

GREENVILLE, NC USA 27858

Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

Current Company Short Name:

Manufacturer/Supplier: U.N.X. INCORPORATED 707 ARLINGTON BLVD. GREENVILLE, NC USA 27858 Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

MSDS Prepared: 6/9/99 MSDS Revised: 4/27/01 Keyword: Specialties Synonym(s): DEGREASER

Physical/Chemical Characteristics

Boiling Point: GT 100   C Specific Gravity: EQ 1.12 H2O=1

pH:BT 12 12.9

Solubility In Water: EXCELLENT Appearance/Odor: LIGHT BROWN LIQUID NO ODOR Physical State: Liquid

****************** DOT INFORMATION:

NAIdNo: 1760

DOT hazard Class: 8

Packing Group: II

Label: CORROSIVE

Proper Shipping Name: COMPOUND, CLEANING LIQUID (SODIUM HYDROXIDE)

Preparers Info.: Amendt, Evans, Kincaid ********************************* COMPOSITION/INFORMATION ON INGREDIENTS

Component Name: SODIUM HYDROXIDE Notes: LD(50)ORAL RAT 140-340 mg/kg Percent: BT    10    20 CAS No.: 1310-73-2 Exposure Limits

OSHA PEL: 2 MG/M3

ACGIH TLV: 2 MG/M3

Component Name: WATER Percent: BT    70    80 CAS No.: 7732-18-5

Exposure Limits

Component Name: MONOETHANOLAMINE

Notes: SARA TITLE 3 SECTION 313 40 CRF LISTED MATERIAL

Percent: LT    5

CAS No.: 141-43-5

EPCRA313: Yes

Exposure Limits

OSHA PEL: 3 PPM    8 MG/M3

OSHA STEL: 6 PPM    15 MG/M3

ACGIH TLV: 3 PPM    7.5 MG/M3

ACGIH STEL: 6 PPM    15 MG/M3

Component Name: SODIUM GLUCONATE Percent: LT    5 CAS No.: 527-07-1 Exposure Limits

Component Name: LAURAMINE OXIDE Percent: BT    0    5 CAS No.: 1643-20-5 Exposure Limits

FIRE AND EXPLOSION DATA

SPECIAL FIRE FIGHTING PROCEDURES: USE NIOSH/MSHA APPROVED SELF CONTAINED

BREATHING APPARATUS WHERE PRODUCT IS INVOLVED IN FIRE.

UNUSUAL FIRE AND EXPLOSION HAZARD: AVOID CONTACT WITH ACIDS AND LAUNDRY

COMPOUNDS. STRONG DECOMPOSITION REACTION CAN RESULT.

AVOID CONTACT WITH ALUMINUM,ZINC,MAGNESIUM,COPPER,ETC. EXPLOSIVE HYDROGEN GAS

CAN BE RELEASED.

REACTIVITY DATA

VERY STABLE. CONDITION TO AVOID: EXCESS MOISTURE CAN CAUSE EXOTHERMIC REACTION. INCOMPATIBILITY: SEE UNUSUAL FIRE AND EXPLOSIVE HAZARDS SECTION. HAZARDOUS POLYMERIZATION WILL NOT OCCUR.

HEALTH HAZARD DATA

ACUTE OR CHRONIC: INHALATION, CAN IRRITATE NOSEJHROAT, AND LUNGS

SKIN CONTACT, CAN CAUSE TISSUE DESTRUCTION.

INGESTION, EXTREMELY HIGH TOXICITY. WILL CAUSE BURNS TO

MOUTH, THROAT.

CARCINOGENICITY: NOT APPLICABLE

SIGNS AND SYMPTOMS OF EXPOSURE: PRODUCT,IS DESTRUCTIVE TO TISSUE. SEVERE BURNS CAN BE PRODUCED. CONTACT WITH EYES CAN PRODUCE PERMANENT DAMAGE. FIRST AID: FOR SKIN, FLUSH THOROUGHLY WITH WATER UNTIL "SLIPPERY" FEELING IS GONE. FOR EYES, FLUSH WITH WARM WATER FOR AT LEAST 15 MINUTES. CONTACT PHYSICIAN. IF INGESTED,DRINK LARGE VOLUMES OF WATER FOLLOWED BY MILD ACIDS SUCH AS 5% ACETIC ACID,ORANGE JUICE,OR ANY CITRUS JUICE. DO NOT INDUCE VOMITING. CALL PHYSICIAN.

PRECAUTIONS FOR SAP E HANDLING AND USE

STEPS FOR SPILL OR RELEASE: FOR SMALL SPILL, IMMEDIATELY HOSE DOWN WITH COOL

WATER AND DISPOSE TO DRAJN.FOR LARGE SPILL, DIKE,COLLECT AND ADD SLOWLY TO LA

VOLUME OF COOL WATER WITH CONSTANT STIRRING. THEN SLOWLY NEUTRALIZE TO pH OF

7.0 WITH MURIATIC OR PHOSPHORIC ACID SOLUTIONS. DISPOSE OF DOWN DRAIN AFTER

APPROVAL OF APPROPRIATE AUTHORITY. CLEAN AND NEUTRALIZE ALL AREAS OF

CONTACT.

WASTE DISPOSAL METHODS: SEE STEPS FOR SPILL OR RELEASE.

HANDLING AND STORING: EMPTY DRUMS MUST BE TREATED AS HAZARDOUS BECAUSE OF

RESIDUES. STORE IN DRY PLACE. DO NOT STORE WITH STRONG ACIDS. KEEP CONTAINER

CLOSED.

CONTROL MEASURES

RESPIRATORY PROTECTION: AVOID BREATHING MIST. IF EXPOSURE LIKELY, USE FILTER

OR PROTECTIVE RESPIRATOR.

MECHANICAL VENTILATION IS ACCEPTABLE.

RUBBER OR PLASTIC GLOVES SHOULD BE WORN.

SAFETY GLASSES OR GOGGLES SHOULD BE WORN.

OTHER PROTECTIVE MEASURES: HAVE EYE BATH AND SAFETY SHOWER CLOSE BY. IF

POTENTIAL FOR SKIN CONTACT, WEAR PROTECTIVE GLOVES AND HAVE AN ACID HAND WASH

NEAR BY(ie. VINEGAR SOLUTION.).

WORK/HYGENIC PRACTICES: WASH HANDS,CHANGE OUT OF CLOTHES AS SOON AS

POSSIBLE. WASH CLOTHES. SHOWER OR BATHE AS SOON AS POSSIBLE.

OTHER INFORMATION

HMIS: HEALTH-2, FLAMMABILITY-0, REACTIVITY-1, SPECIAL-CORR

PERSONAL PROTECTION - C

UNLESS OTHERWISE NOTED, NO COMPONENTS ARE SARA TITLE 3 SECTION 313 40 CFR LISTED MATERIALS

THIS PRODUCT CONTAINS NO PHOSPHATES

NOT MADE WITH VOC'S THAT COULD CAUSE DAMAGE TO THE OZONE LAYER THE INGREDIENTS OF THIS PRODUCT ARE LISTED ON THE TSCA INVENTORY SHIPPING INFORMATION: COMPOUND, CLEANING LIQUID, INORGANIC, BASIC, (SODIUM HYDROXIDE) PG II

END OF REPORT.