LVALUE

P.O. BOX 2235 Smyrna, Georgia 30081

MATERIAL SAFETY DATA SHEET

Meeting OSHA Standard 29 CFR §1910.1 200 (g)

EFFECTIVE DATE:

Nov. 25, 1985

24 Hour Emergency telephone number: 404-588-4400

SECTION I — PRODUCT IDENTIFICATION

PRODUCT TRADE NAME: Mop-up

CHEMICAL NAME AND SYNONYMS: Disodium Octaborate Tetrahydrate

CHEMICAL FAMILY: Sodium borate
HAZARD RATING: NFPA: Health	0

Flammability   0

Reactivity         "0

CAS NO: 12008-41-2

FORMULA: Na2Bflo,3.4H20

SECTION II — HAZARDOUS INGREDIENTS

Material or Component

None

Material or Component

SECTION III — PHYSICAL DATA

Approx. Formula weight 412 White, odorless powder (light blue tint) Bulk density 25-35 Ibs/ft3 Particle Size: 149-44 microns pH@23°C(73.4°F)    1% 8.5

10% 7.6

15% 7.3

Solubility in water:

20°C    (68°F) 9.5%

940C(201°F) 45.3%

SECTION IV —HEALTH HAZARD INFORMATJON

EFFECTS OF EXPOSURE (SYMPTOMS) INGESTION: Nausea, vomiting.

ACTUE ORAL LD60 2.0 g/kg body weight (rats). EYE: (16CFR §1500.42) May be slightly irritating. Reversible. SKIN: Not an irritant.

ACUTE DERMAL LD50 (1 6CFR §1 500.40) Unknown PRIMARY SKIN IRRITATION INDEX; (1 6CF §1 500.41) Less than 1. CORROSIVE

INHALATION May be irritating to nose and throat, ACUTE LC50 Unknown.

NON CORROSIVE: X per 49 CFR §240

EFFECTS OF OVEREXPOSURE

ACUTE OVEREXPOSURE/METHOD OF EXPOSURE/SYMPTOMS

Eyes — may be irritating. Reversible. Skin — None expected, Mouth/Nose — May be slightly irritating. Ingestion — Nausea/vomiting.

CHRONIC OVEREXPOSURE/METHOD OF EXPOSURE/SYMPTOMS

Eyes: May be irritating.

Skin: May be irritating on prolonged exposure on wet skin.

Mouth/Nose: May be irritating.

Ingestions: Nausea, vomiting, diarrhea, skin rash.

POLYBOR® 11/1/85