AIR61505170

maintenance supply company, inc.

post office box 498 huntersville, north carolina 28070

- material safety data sheet -

these material safety data sheets are sent to you in compliance with 29 cfr 1910.1200 which became effective november 25, 1985. medical emergency only, 24 hour service: 1-800-328-0026 medical calls from outside of the usa: 1-651-222-5352 product information: 1-800-247-5362 distributor's msds reference:   66500 manufacturer's reference: 61507560, 62380101 revision date:   9/17/1999 manufactured by:  professional products

ecolab, inc.

370 wabasha street North St. Paul, MN 55102

1.0  IDENTIFICATION /

- Product Name: QUIK fill 530

- Product Type: Heavy Duty Non-Butyl/Degreaser

- Hazard Rating: Health: 3    Fire: 0    Reactivity: 0

Substances subject to sara 313 Reporting Are indicated by "#"

2.0	HAZARDOUS COMPONENTS / .	(mg/m3)

%      PEL     TWA

- Sodium hydroxide (caustic soda)        2      2     2C
(CAS # 1310-73-2)

- Sodium metasilicate	1-5     No     No
(CAS # 6834-92-0)

- Ethanolamine (MEA)	5-20     6     7.5*
(CAS # 141-43-5) '	-ACGIH STEL = 15

- chelating agents	1-5     No      No
(CAS # confidential)

- Nonionic surfactants	1-5     No      No
(CAS # confidential)

- Anionic surfactants	1-5     No     No
(CAS # confidential)

stel = ACGIH short term exposure limit (15 min)

PEL = osha 8 Hour Average in Air

TWA = acgih 8 Hour Average

c = ceiling limit in air, do not exceed

3.0	PHYSICAL DATA /

- Appearance: Clear to slightly hazy yellow/green liquid; lemon
scent.

- solubility in Water: Mixes with water in all proportions.

- pH: 13.0 - 13.5   (100%);     12.3 - 12.8  (10%)

- initial   Boiling Point:     212 deg.   F.

- Specific Gravity:     1.070 @ 68 deg.   F.

4.0  FIRE AND EXPLOSION DATA /

r

- Special Fire Hazards: None

- Fire Fighting Methods: Does not support combustion.

5.0  REACTIVITY DATA /

Page 1

AIR61505170

- Stability:  Stable under	normal conditions 9f handling.

- Conditions to Avoid: Do	not mix with anything but water.
Keep from freezing.

6.0 SPILL OR LEAK PROCEDURES	/ USE PROPER PROTECTIVE EQUIPMENT

- Cleanup:  Dike or dam large	spills.  Pump to containers or
soak up on inert absorbent.	Flush residue to sanitary
sewer.

- Waste Disposal: Consult state/local authorities for limits
on chemical waste disposal,	unused product is RCRA Corrosive
(D002).

7.0  HEALTH HAZARD DATA /	DANGER

7.1 Effects of Overexposure to Concentrate:

Eyes: causes chemical burns. May cause blindness. ++ immediate water flushing is vital in case of eye contact. ++ Skin: causes chemical BURNS.  Harmful contact may not

cause immediate pain, if swallowed: harmful or fatal, causes chemical burns of

mouth, throat and stomach.

If Inhaled:  Causes irritation to bronchitis or pneumonia, depending on exposure.  People with lung problems may be more affected.

8.0 FIRST AID /

8.1	Eyes: Flush at once with cool running water.  Remove

contact lenses and continue flushing for 15 minutes, holding eyelids apart so as to rinse entire eye. CALL A

PHYSICIAN IMMEDIATELY.

8.2	skin: immediately flush skin with plenty of cool running

water for at least 15 minutes while removing contaminated clothing and shoes. Discard or wash before reuse.

8.3	if Swallowed:  Rinse mouth at once; then drink 1 or 2 large

glasses of water. DO NOT induce vomiting. never give anything by mouth to an unconscious person.

8.4	if inhaled: immediately move to fresh air.

IMMEDIATELY CALL THE MEDICAL EMERGENCY NUMBER, 1-800-328-0026, A POISON CONTROL CENTER, OR A PHYSICIAN

9.0	PROTECTIVE MEASURES/

9.1	FOR CONCENTRATE:

Eyes:  wear chemical splash goggles.  For continued or

severe exposure wear a face shield over the goggles. Skin: wear any industrial rubber gloves, other protective

equipment as necessary to prevent skin contact. Respiratory: Ventilate to maintain air levels below

exposure limits. Avoid breathing mists or

vapors of this product.

10.0  ADDITIONAL INFORMATION /

-2-

10.1 Purpose of 09/17/1999 issue:  Periodic review; revise

section 2, QC and editorial update.  Replaces January 7, 1998 issue.

KEEP pUT OF REACH OF CHILDREN

The above information is believed to be correct with respect to

Page 2