MATERIAL SAFETY DATA SHEET

Revised: 16/07/04

SUDAN III FAT STAIN	Replaces: 09/05/00

CAROLINA BIOLOGICAL	Printed: 20/05/08

1.	PRODUCT DESCRIPTION

Product Name:      Sudan III Fat Stain

Product Code(s):   89-2963, 89-2965

Size:	120 ml, 500 ml

Chemical Name:     Sudan III Fat Stain

CAS Number:        See section 2

Formula:	See section 2

Synonyms:         None

Distributor:       Carolina Biological Supply Company

2700 York Road

Burlington, NC 27215

Chemical Information:  800-227-1150 (8am-5pm (ET) M-F) Chemtrec (Transportation Spill Response 24 hours): 800-424-9300

2.	COMPOSITION/INFORMATION ON INGREDIENTS

Principle Hazardous Components:

Methyl isobutyl ketone (denaturant)(cas#108-10-1)1%

Hexane (denaturant)(cas#110-54-3)1%

Acetone (cas#67-64-1)50%

Ethyl alcohol(cas#64-17-5)48%

Sudan III (cas#85-86-9)1%

TLV and PEL units:

Acetone:	ACGIH-TLV 750 ppm(TWA), STEL 1000 ppm

OSHA-PEL 750 ppm (TWA), STEL 1000 ppm
Sudan III:	None

Ethyl Alcohol:	ACGIH TLV 1000 ppm (TWA)

OSHA-PEL 1000 ppm (TWA) Methyl isobutyl ketone:  ACGIH-TLV 50 ppm (TWA), STEL 75 ppm

OSHA-PEL 50 ppm (TWA), STEL 75 ppm
Hexane:	ACGIH-TLV 50 ppm (TWA)

OSHA-PEL 50 ppm (TWA)

3.	HAZARD IDENTIFICATION

Emergency Overview:  Harmful if ingested.  Dark red liquid with

characteristic odor of ethyl alcohol and acetone.  May cause

dizziness.  May be irritating to eyes, nose and throat.

Potential Health Effects:

Eyes:  May cause irritation.

Skin:  May cause irritation.

Ingestion:  May cause gastrointestinal discomfort.

Inhalation:  May cause irritation to respiratory tract.

4.	FIRST AID MEASURES

Emergency and First Aid Procedures:

Eyes - Flush with water for at least 15 minutes, raising and lowering

eyelids     occasionally.  Get medical attention if irritation

persists.

Skin - Thoroughly wash exposed area for at least 15 minutes.  Remove

contaminated clothing.  Launder contaminated clothing before reuse.  Get

medical attention if irritation persists.

Ingestion - If swallowed, if conscious, give plenty of water and induce

vomiting immediately as directed by medical personnel.  Immediately call

a physician or poison control center. Never give anything by mouth to an

unconscious person.

Page 1 of 4

MATERIAL SAFETY DATA SHEET

Revised: 16/07/04

SUDAN III FAT STAIN	Replaces: 09/05/00

CAROLINA BIOLOGICAL	Printed: 20/05/08

Inhalation - Remove to fresh air.  Give oxygen if breathing is difficult; give artificial respiration if breathing has stopped.  Keep person warm, quiet, and get medical attention.

5.	FIREFIGHTING PROCEDURES

Flash Point(Method Used):  (CC) 0 F-Acetone NFPA Rating:      Health:  2

Fire:  3

Reactivity:  0 Extinguisher Media:

Use dry chemical, C02 or appropriate foam.

Flammable' Limits in Air % by Volume:  Acetone:  LEL 2.6%, UEL 12.8%, Ethyl

alcohol:  LEL 3.3%, UEL 19%

Autoignition Temperature:  Acetone 869 F; Ethyl alcohol 793 F Special Firefighting Procedures:

Firefighters should wear full protective equipment and NIOSH approved self-contained breathing apparatus. Unusual Fire and Explosion Hazards:

Explosion hazard when vapor is exposed to flame in confined space. Dangerous fire hazard when exposed to heat, flame, or oxidizers.

6.	SPILL OR LEAK PROCEDURES

Steps to be Taken in Case Material is Released or Spilled:

Ventilate area of spill.  Eliminate all sources of ignition. Remove all non-essential personnel from area.  Clean-up personnel should wear proper protective equipment and clothing.  Absorb material with suitable absorbent and containerize for disposal.

7.	SPECIAL PRECAUTIONS

Precautions to be Taken in Handling or Storing:  This material should be

kept

in an area suitable for the storage of highly flammable liquids.  Bond

and

ground containers when transferring liquid.  Do not take internally.

Other Precautions:  Keep oxidizing materials and strong acids away.

Wear

splash-proof safety goggles and rubber gloves at all times during

cleanup

operations.  Wash thoroughly after handling.

8.	SPECIAL PROTECTION INFORMATION

Respiratory Protection(Specify Type):

A NIOSH/MSHA chemical cartridge respirator should be worn if PEL or TLV is exceeded. Ventilation:

Local Exhaust: Yes
Mechanical(General): Yes
Special:	No

Other:	No

Protective Gloves: Butyl, rubber or equivalent Eye Protection:

Splash proof chemical safety goggles should be worn at all times. Other Protective Clothing or Equipment:

Page 2 of 4

MATERIAL SAFETY DATA SHEET

Revised: 16/07/04

SUDAN III FAT STAIN	Replaces: 09/05/00

CAROLINA BIOLOGICAL	Printed: 20/05/08

Lab apron, eye wash, and safety shower.

9.	PHYSICAL DATA

Molecular Weight:	N/A

Melting Point:	N/A

Boiling Point:	N/A

Vapor Pressure:	N/A

Vapor Density(Air=l):	Greater than 1

Specific Gravity(H20=l):	N/A

Percent Volatile by Volume:	Greater than 99%

Evaporation Rate(H20=l):	Greater than 1

Solubility in Water:	Miscible

Appearance and Odor:	Dark red liquid with characteristic odor of

ethyl alcohol and acetone.

10.	REACTIVITY DATA

Stability:  Stable

Conditions to Avoid:  Heat, sparks and open flame

Incompatibility(Materials to Avoid):  Oxidizing materials, chloroform

plus

alkalis, acids.

Hazardous Decomposition Products:  Carbon dioxide, carbon monoxide

Hazardous Polymerization:  Will not occur

11.TOXICITYDATA

Toxicity Data:

Effects of Overexposure:

Acute:  See section 3

Chronic:

Conditions Aggravated by Overexposure:  Pre-existing conditions of the eyes,

skin, kidneys, or liver.

Target Organs:  Eyes, skin, kidneys, liver Primary Route(s) of Entry:  Inhalation, ingestion,

12.	ECOLOGICAL DATA

EPA Waste Numbers:  U002, U161

13.	DISPOSAL INFORMATION

Waste Disposal Methods:  Dispose in accordance with all applicable

Federal,

State and Local regulations.

Always contact a permitted waste disposer (TSD) to assure compliance.

14.	TRANSPORT INFORMATION

Description: Flammable liquid, n.o.s. (acetone, ethanol, Methyl isobutyl ketone), 3, UN1993, II

15.	REGULATORY INFORMATION

EPA TSCA Status:  On TSCA Inventory

Hazard Category for SARA Section 311/312 Reporting:  Acute

SARA Sec. 313

SARA EHS      Chemicals	CERCLA      RCRA

Product or       Sec. 302      Name  Chemical	Sec. 103    Sec.

Components        TPQ        List  Category         RQ Ibs. 261.33

Page 3 of 4